Edge-Computing Video Analytics for Real-Time Traffic Monitoring in a Smart City
 Analysis of the Liverpool Smart Pedestrians Project

Brandy Griffin

ITAI 3377 – AI at The Edge & IIoT Environments

Professor: Sitaram Ayyagari

February 7, 2026

# A. Introduction and Objectives

The Liverpool Smart Pedestrians project uses video analytics on the edge to help with the city's traffic problems that are getting worse. The project's main goal was to make and test a smart visual sensor that could find and follow people, cars, and bikes in real time without invading their privacy.

Cities are still growing quickly, which makes it hard to get around, plan, and deal with traffic jams. A lot of cities already have big networks of CCTV cameras, but because of privacy laws, these systems are mostly only used to look at things that have already happened. Because of this, a lot of helpful video data is still not being used. The goal of this project was to make regular CCTV systems smarter by processing video in real time and sending only useful metadata instead of raw footage.

The main goal was to give people accurate, up-to-date information about mobility while keeping their privacy and making the network less busy. This would help people use data to make better decisions about how to plan cities.

# B. Methodology

The project started with workshops in the community and meetings with city planners to figure out what people really needed. Four important needs came up in those meetings:

Keeping track of and finding a lot of things, like people, cars, and bikes.

Following the rules about keeping private information private

Making the most of the current CCTV system

The ability to grow and work with other systems

The sensor goes around in a circle about 20 times a second. Every time it does this:

Takes a picture from a live video stream

Finds things that matter

Follows those things from one frame to the next.

Changes the trajectory records that are stored on the device

Sends metadata that has been shortened

One of the most important design choices was making sure that no raw video could leave the device. Only processed data, like counts of objects and patterns of movement, is sent. This protects user privacy and reduces bandwidth usage.

We also thought about real-life limits. The device must be able to work outside, follow LoRaWAN communication rules, and have only 32 GB of built-in storage. These restrictions had a direct effect on the hardware and algorithms that were chosen.

# C. Technology and Implementation

The NVIDIA Jetson TX2 is a computer platform that is already in other devices. It has a 256-core GPU and 8 GB of memory. This is the most important part of the sensor. This means that you can use deep learning inference on your own computer in real time without having to use the cloud.

The device is protected from the weather and has Ethernet and LoRaWAN communication modules. Processing data at the edge cuts down on latency, uses less bandwidth, and keeps private video data in the current CCTV system.

YOLO V3 for Looking for Things

YOLO v3 is used by the system to find things. YOLO was chosen because it is quick and accurate, which makes it a good choice for real-time monitoring.

The detection model was configured with:

Resolution of the input is 416 × 416

A level of confidence of 0.9

The limit for non-maximum suppression is 0.5.

There are six object classes: pedestrian, bicycle, car, bus, truck, and motorbike.

When running on the Jetson TX2 GPU, detection works at speeds that are almost real time.

SORT: Following Things

Once the system finds an object, it uses the SORT algorithm to keep track of it from frame to frame. Using a Kalman filter and the Hungarian algorithm, SORT quickly finds matches between objects in frames.

It was very important to keep track of the following:

Minimum number of hits: 3

Age limit: 40

Threshold for IoU: 0.3

This method keeps track of things without using as much processing power.

Agnosticity Framework

The Agnosticity framework is the IoT data management platform that gathers metadata from the sensors. It works with other systems, so different sensors can use the same ways to talk to each other. This lets city planners store data for a long time and view it on a dashboard.

# D. Validation and Performance

We used the Oxford Town Center dataset, which had 4,500 labeled frames of pedestrian data, to test the system.

The results of the tests showed that:

On average, 69% right

33% median relative mistake

The average processing speed is 19.57 frames per second.

A tendency to underestimate object counts

Most of the time, reduced accuracy occurred in crowded scenes due to occlusion and overlapping bounding boxes. The more people there were, the less accurate it became.

Performance analysis also showed that processing frames took longer when there were more objects found. The processing took longer because the CPU was running the tracking algorithm.

In the real world, the system used memory and disk space all the time, the GPU was used a lot during inference, and there was a lot less outbound bandwidth than inbound video streams. This shows that processing at the edge makes the network less busy.

# E. Real-World Applications

The sensor was put in a college building for an hour. At this point:

There were 631 people who were found and followed.

There were two clear spikes in movement during the fire alarm.

The alarm went off, and the system knew that everyone had left.

The system used heat maps and trajectory mapping to see how people moved and to find patterns in how crowds behaved.

Deployment outside—Liverpool

There are now twenty sensors in the center of Liverpool.

People saw this on one day:

There were 20,399 unique things that were discovered.

Every day, the traffic followed clear patterns.

People could walk in two main ways.

The data showed that traffic patterns changed throughout the day, with the busiest times being during the day when most people were at work. These results show that the system can keep track of how people move around in the real world and help city planners make decisions based on that information.

# F. Challenges and Future Work

There were some issues:

Detection accuracy is only 69%

Reduced accuracy in crowded scenes due to occlusion and overlapping detections

The tracking algorithm is making the CPU run slower.

The Jetson TX2's hardware performance isn't perfect.

Future improvements will include using more powerful embedded hardware, making deep learning frameworks work better, and getting rid of bottlenecks by using GPU-based tracking.

If these changes were made on a large scale, they would probably make things go faster and be more accurate.

# G. Personal Evaluation

I think the Liverpool Smart Pedestrians project is a new and useful way to make cities smarter. It's cheap and good for privacy to make current CCTV systems into smart edge devices.

The system's detection wasn't perfect, but it was able to find patterns in traffic and changes in how people move around. It can be more useful for city planning to see consistent flow patterns than to get exact counts of things.

If I could make this system better, I would work on making tracking faster with GPU resources, making edge devices safer from cyber attacks, and adding more types of sensors to make it easier to find things when things get complicated.

The project demonstrates the integration of edge computing, artificial intelligence, and the Internet of Things (IoT) to create urban monitoring systems that can expand while safeguarding individual privacy.

# References

Barthélemy, J., Verstaevel, N., Forehead, H., & Perez, P. (2019). Edge-computing video analytics for real-time traffic monitoring in a smart city. Sensors, 19(9), 2048. https://doi.org/10.3390/s19092048